SPECIFICATIONS: HYDRA-MATIC 6L80 

	 
	2007 Hydra-Matic 6L80 (MYC)  

	Type: 
	Six-speed RWD / AWD, electronically controlled automatic overdrive transmission with torque converter clutch. Clutch-to-clutch architecture, with integral Electro / Hydraulic Controls Module 

	Maximum engine power: 
	413 bhp (308 kW) 

	Maximum engine torque: 
	430 lb-ft (583 Nm) 

	Maximum gearbox torque: 
	664 lb-ft (900 Nm) 

	Gear ratios: 
	MYC 

	First: 
	4.03 

	Second: 
	2.36 

	Third: 
	1.53 

	Fourth: 
	1.15 

	Fifth: 
	0.85 

	Sixth: 
	0.67 

	Reverse: 
	-3.06 

	Maximum shift speed 
	6500 rpm 

	Maximum validated weights:(target ) 
	GVW: 8600 lb (3901 kg), GCVW: 14000 lb (6350 kg) 

	7-position quadrant: 
	P, R, N, D, X, X, X (X = available calibratable range position ) 

	Case description: 
	Three-piece ( Bell, main, extension) 

	Case material: 
	die cast aluminum 

	Shift pattern: 
	(2) Three-way on / off solenoids 

	Shift quality: 
	Five variable bleed solenoids 

	Torque converter clutch: 
	Variable Bleed Solenoid ECCC 

	Converter size: 
	300 mm ( reference ) 

	Fluid type: 
	DEXRON® VI 

	Fluid capacity: w/ 258 & 300mm converter 
	258 mm: 9.7 L (8.22 kg), 300 mm: 11.9 L (10.1 kg) 

	Transmission weight: w/ 258 & 300mm converter 
	258 mm: Wet: 94-96 kg (207-211 lb) 

	 
	300 mm: Wet: 102-104 kg (225-229 lb) 

	Pressure taps available: 
	line pressure 

	Assembly site: 
	GMPT Ypsilanti, Mich. 

	 
	GMPT Toledo, Mich. (production startup 2008) 

	Applications: 
	Chevrolet Corvette 

	
	Cadillac XLR 

	
	Cadillac XLR-V

	
	Cadillac STS-V

	
	GMC Yukon Denali 

	
	GMC Yukon Denali XL 

	
	Cadillac Escalade 

	
	Cadillac Escalade ESV 

	
	Cadillac Escalade EXT 

	Available Control Features: 
	Multiple shift patterns (selectable or adaptive) 

	 
	Driver Shift Control (Tap Up / Tap Down) 

	 
	Enhanced Performance Algorithm Shifting (PAS II) 

	 
	Next-generation tow / haul mode 

	 
	Engine torque management on all shifts 

	 
	Altitude and temperature compensation 

	 
	Adaptive shift time 

	 
	Neutral idle 

	 
	Reverse lockout 

	 
	Automatic grade braking 

	Additional Features: 
	GM Oil Life System 

	 
	OBDII / EOBD 

	 
	Integral Electro/Hydraulic Controls Module – TEHCM 

	 
	Control Interface Protocol - GMLAN 


