Fig. 2001-02 C1 Connector

| Pin | Wire Color | Circuit No. | Function |
| --- | --- | --- | --- |
| 1 | BLK/WHT | 451 | Ground |
| 2 | LT GRN | 1867 | 12 Volt Reference |
| 3 | PNK/BLK | 1746 | Fuel Injector 3 Control |
| 4 | LT GRN/BLK | 1745 | Fuel Injector 2 Control |
| 5-10 | -- | -- | Not Used |
| 11 | LT BLU | 1876 | Knock Sensor 2 Signal |
| 12 | DK BLU/WHT | 1869 | CKP Sensor Signal |
| 13 | ORN/BLK | 463 | Requested Torque Signal |
| 14 | ORN/BLK | 1061 | UART Serial Data [Secondary] |
| 15 | DRK BLU/WHT | 774 | UART Serial Data [Tertiary] |
| 16 | -- | -- | Not Used |
| 17 | DK BLU | 1225 | Transmission Fluid Pressure Switch Signal B |
| 18 | RED | 1226 | Transmission Fluid Pressure Switch Signal C |
| 19 | PNK | 439 | Ignition 1 Voltage |
| 20 | ORN | 440 | Battery Positive Voltage |
| 21 | YEL | 1868 | Low Reference |
| 22 | -- | -- | Not Used |
| 23 | PPL | 719 | Low Reference |
| 24 | -- | -- | Not Used |
| 25 | TAN | 1671 | HO2S Low Signal [- Bank 2 Sensor 1] |
| 25 | TAN | 413 | HO2S Low Reference (Denso Sensors) HO2S Low Signal (Delphi Sensors) |
| 26 | TAN | 1667 | HO2S Low Signal [- Bank 2 Sensor 2] |
| 26 | TAN | 413 | HO2S Low Reference (Denso Sensors) HO2S Low Signal (Delphi Sensors) |
| 27 | -- | -- | Not Used |
| 28 | TAN/WHT | 1669 | HO2S Low Signal [- Bank 1 Sensor 2] |
| 28 | TAN | 413 | HO2S Low Reference (Denso Sensors) HO2S Low Signal (Delphi Sensors) |
| 29 | TAN/WHT | 1653 | HO2S Low Signal [- Bank 1 Sensor 1] |
| 29 | TAN | 413 | HO2S Low Reference (Denso Sensors) HO2S Low Signal (Delphi Sensors) |
| 30 | LT GRN | 1478 | Coolant Level Switch Signal |
| 31 | -- | -- | Not Used |
| 32 | BLK/WHT | 771 | Transmission Range Switch Signal A |
| 33 | PPL | 420 | TCC Brake Switch Signal |
| 34 | WHT | 776 | Transmission Range Switch Signal P |
| 35 | GRA | 48 | CPP Switch Signal |
| 36 | BLK | 1744 | Injector 1 Control |
| 37 | YEL/BLK | 846 | Injector 6 Control |
| 38 | PNK/WHT | 1101 | Damping Lift/Dive Signal |
| 39 | -- | -- | Not Used |
| 40 | BLK/WHT | 451 | Ground |

2001-02 4.8L, 5.3L, 6.0L Engine PCM C1 Connector Pinout (1 of 2)

Fig. 2001-02 C1 Connector (Continued)

| Pin | Wire Color | Circuit No. | Function |
| --- | --- | --- | --- |
| 41 | BLK | 407 | Low Reference |
| 41 | GRA | 720 | Low Reference (w/o Delphi HO2S) |
| 42 | -- | -- | Not Used |
| 43 | RED/BLK | 877 | Fuel Injector 7 Control |
| 44 | LT BLU/BLK | 844 | Fuel Injector 4 Control |
| 45 | GRA | 474 | 5 Volt Reference (FTP Sensor) |
| 46 | GRY | 474 | 5 Volt Reference (TPS Sensor) |
| 47 | GRY | 597 | 5 Volt Reference (MAP Sensor) |
| 48 | GRA | 596 | 5 Volt Reference (EGR Sensor) |
| 4950 | -- | -- | Not Used |
| 51 | DK BLU | 496 | KS [1] Signal |
| 52 | -- | -- | Not Used |
| 53 | BLK | 470 | Low Reference |
| 54 | BLK | 452 | Low Reference |
| 55 | BRN | 1456 | EGR Valve Position Signal |
| 56 | -- | -- | Not Used |
| 57 | ORN | 440 | Battery Positive Voltage |
| 58 | DK GRN | 1049 | ECM/PCM/VCM Class 2 Serial Data |
| 59 | YEL | 710 | Class 2 Serial Data |
| 60 | ORN/BLK | 469 | Low Reference |
| 61 | PNK/BLK | 632 | Low Reference |
| 62 | -- | -- | Not Used |
| 63 | GRA | 720 | Low Reference |
| 63 | TAN | 413 | HO2S Low Reference (Denso Sensors) HO2S Low Signal (Delphi Sensors) |
| 64 | -- | -- | Not Used |
| 65 | PPL | 1670 | HO2S High Signal [- Bank 2 Sensor 2] |
| 66 | PPL | 1666 | HO2S High Signal [- Bank 2 Sensor 1] |
| 67 | -- | -- | Not Used |
| 68 | PPL/WHT | 1668 | HO2S High Signal [- Bank 1 Sensor 2] |
| 69 | PPL/WHT | 1665 | HO2S High Signal [- Bank 1 Sensor 1] |
| 70 | BRN | 1174 | Oil Level Switch Signal |
| 71 | -- | -- | Not Used |
| 72 | YEL | 772 | Transmission Range Switch Signal B |
| 73 | BRN/WHT | 633 | CMP Sensor Signal |
| 74 | YEL | 410 | ECT Sensor Signal |
| 75 | PNK | 1020 | Off/Run/Crank Voltage |
| 76 | BLK/WHT | 845 | Fuel Injector 5 Control |
| 77 | DK BLU/WHT | 878 | Fuel Injector 8 Control |
| 78 | -- | -- | Not Used |
| 79 | WHT | 687 | 3-2 Shift Solenoid Valve Control |
| 80 | ORN/BLK | 510 | Low Reference |

2001-02 4.8L, 5.3L, 6.0L Engine PCM C1 Connector Pinout (2 of 2)

Fig. 2001-02 C2 Connector

| Pin | Wire Color | Circuit No. | Function |
| --- | --- | --- | --- |
| 1 | BLK/WHT | 451 | Ground |
| 2 | BRN | 418 | TCC PWM Solenoid Valve Control |
| 3 | TAN | 1465 | Fuel Pump Relay Control [- Secondary] |
| 4 | -- | -- | Not Used |
| 5 | TAN/BLK | 464 | Delivered Torque Signal |
| 6 | RED/BLK | 1228 | PC Solenoid Valve High Control (Sol. A) |
| 7 | RED | 1676 | EGR Valve Supply Voltage (5 Volt) |
| 8 | LT BLU/WHT | 1229 | PC Solenoid Valve Low Control (Sol. A) |
|  | 9 | DK GRN/WHT | 465 | Fuel Pump Relay Control [- Primary] |  |
|  | 10 | WHT | 121 | Engine Speed Signal |  |
|  | 11 | DK BLU | 604 | A/C High Pressure Recirculation Switch Signal |  |
|  | 12 | -- | -- | Not Used |  |
|  | 13 | LT BLU/BLK | 396 | Cruise Control Engaged Signal |  |
|  | 14 | -- | -- | Not Used |  |
|  | 15 | BRN | 25 | Charge Indicator Control |  |
|  | 16 | GRY/BLK | 1694 | 4WD Low Signal |  |
|  | 17 | DK GRN/WHT | 2523 | A/C Request |  |
|  | 18 | -- | -- | Not Used |  |
|  | 19 | BLK/WHT | 1695 | Axle Switch Signal |  |
|  | 20 | LT GRN/BLK | 822 | VSS Low Signal |  |
|  | 21 | PPL/WHT | 821 | VSS High Signal |  |
|  | 22 | RED/BLK | 1230 | AT ISS High Signal |  |
|  | 23 | DK BLU/WHT | 1231 | AT ISS Low Signal |  |
|  | 24 | -- | -- | Not Used |  |
|  | 25 | TAN | 472 | IAT Sensor Signal |  |
|  | 26 | PPL | 2121 | IC 1 Control |  |
|  | 27 | RED | 2127 | IC 7 Control |  |
|  | 28 | LT BLU/WHT | 2126 | IC 6 Control |  |
|  | 29 | DK GRN/WHT | 2124 | IC 4 Control |  |
|  | 30 | -- | -- | Not Used |  |
|  | 31 | YEL | 492 | MAF Sensor Signal |  |
|  | 32 | LT GRN | 432 | MAP Sensor Signal |  |
|  | 33 | DK GRN | 1614 | Recirculation Actuator Control |  |
|  | 34 | DK GRN/WHT | 428 | EVAP Canister Purge Solenoid Control |  |
|  | 35 | -- | -- | Not Used |  |
|  | 36 | BRN | 436 | AIR Pump Relay Control |  |
|  | 37-38 | -- | -- | Not Used |  |
|  | 39 | RED | 631 | 12 Volt Reference |  |
|  | 40 | BLK/WHT | 451 | Ground |  |
|  | 41 | WHT | 257 | EGR Solenoid Control |  |
|  | 42 | TAN/BLK | 422 | TCC Solenoid Valve Control |  |
|  | 43 | DK GRN/WHT | 459 | A/C Compressor Clutch Relay Control |  |

2001-02 4.8L, 5.3L, 6.0L Engine PCM C2 Connector Pinout (1 of 2)

Fig. 2001-02 C2 Connector (CONTINUED)

| Pin | Wire Color | Circuit No. | Function |
| --- | --- | --- | --- |
| 44 | -- | -- | Not Used |
| 45 | WHT | 1310 | EVAP Canister Vent Solenoid Control |
| 46 | BRN/WHT | 419 | MIL Control |
| 47 | YEL/BLK | 1223 | (2-3 Shift Solenoid or Shift Soleniod B) Valve Control |
| 48 | LT GRN | 1222 | (1-2 Shift Solenoid or Shift Soleniod A) Valve Control |
| 49 | YEL/BLK | 1827 | Vehicle Speed Signal |
| 50 | DK GRN/WHT | 817 | VSS Signal |
| 51 | YEL/BLK | 1227 | TFT Sensor Signal |
| 52 | GRA | 23 | Generator Field Duty Cycle Signal |
| 53 | -- | -- | Not Used |
| 54 | PPL | 1589 | Fuel Level Sensor Signal [- Primary] |
| 55 | DK GRN | 603 | A/C Low Pressure Switch Signal |
| 56 | -- | -- | Not Used |
| 57 | BLK | 552 | Low Reference |
| 58-59 | -- | -- | Not Used |
| 60 | BRN | 2129 | Low Reference |
| 61 | BRN/WHT | 2130 | Low Reference |
| 62 | GRA | 773 | Transmission Range Switch Signal C |
| 63 | PNK | 1224 | Transmission Fluid Pressure Switch Signal A |
| 64 | DK GRN | 890 | Fuel Tank Pressure Sensor Signal |
| 65 | -- | -- | Not Used |
| 66 | PPL/WHT | 2128 | IC 8 Control |
| 67 | RED/WHT | 2122 | IC 2 Control |
| 68 | DK GRN | 2125 | IC 5 Control |
| 69 | LT BLU | 2123 | IC 3 Control |
| 70-71 | -- | -- | Not Used |
| 72 | BLK/WHT | 3122 | HO2S Heater Low Control [- Bank 2 Sensor 1] |
| 73 | DK BLU | 1936 | Fuel Level Sensor Signal [- Secondary] |
| 74 | LT GRN/WHT | 3213 | HO2S Heater High Control [- Bank 2 Sensor 1] |
| 75 | -- | -- | Not Used |
| 76 | LT GRN/WHT | 1749 | IAC Coil B High Control |
| 77 | LT GRN/BLK | 444 | IAC Coil B Low Control |
| 78 | LT BLU/BLK | 1748 | IAC Coil A Low Control |
| 79 | LT BLU/WHT | 1747 | IAC Coil A High Control |
| 80 | LT GRN | 3212 | HO2S Heater Low Control [- Bank 2 Sensor 1] |

2001-02 4.8L, 5.3L, 6.0L Engine PCM C2 Connector Pinout (2 of 2)